Preflight Inspection

Bottom Comm #2 Antenna

Left Fuel Vent - Clear

Left Wing

Left Fuel Tank – Check Quantity

Left Fuel Tank – Cap Secure

Pitot Tube

Left Landing Light

Left Wing Tip / Lights

Left Aileron

Left Flap & Actuator Rod

Left Wheel Condition / Brake / Fluid Leak

Left Static Port

Top Antennas (3)

Left Horizontal Stab

Left Elevator

Trim Tab / Clevis

Rudder / Cables / Light

Right Elevator

Right Horizontal Stab

Right Static Port

Right Wheel Condition / Brake / Fluid Leak

Right Flap & Actuator Rod

Right Aileron

Right Wing Tip / Lights

Right Landing Light

Right Fuel Tank – Check Quantity

Right Fuel Tank – Cap Secure

Right Wing

Oil Quantity – Check

Interior Fluid Leak Check

Oil Cap / Oil Door – Close / Secure

Right Fuel Vent - Clear

Cowl Secure

Propeller – Check for nicks

Spinner – Check Secure

Air intakes - Clear

Air Filter – Check Clean

Nose Gear – Check Inflation / Condition

Before Start:

Preflight Inspection – Complete

Test Brakes

AROW Documents On Board

Goggles On Seat

Hatchet On Board

Fire Extinguisher On Board

Fuel – 20+ Gallons On Board

Avionics Switch – Off

Starting:

Canopy Closed & Both Latches Secured

Fuel Selector – Left Tank (Or most full tank)

Throttle – ¼ Open

Prop – High RPM

Master – On

Flaps up

Fuel Quantity - Check

Fuel Pump – On

Mixture - Rich

Fuel Flow – Watch for rise & stabilize

Mixture – Lean

Fuel Pump – Off

Brakes - Set

Announce – Clear Prop

Left Mag – On

Starter – Engage

Right Mag – On

Alternator – On

Oil Pressure – Verify

Nav Lights – On

Strobe Lights – As Desired

RPM – 1000

Avionics – On

Headset – On

CO Monitor – Test

Mixture – Lean 1”

Before Taxi:

Radio #2: AWOS – 125.175

Altimeter - Set

Comm#1: Ground Ctrl – 121.8 TX&RX

Comm#2: Ground Crew – 123.45 RX

Transponder: 1234

GPS Direct to KHEF

Before Takeoff:

Canopy – Both Latches Secure

Fuel Quantity – Check

Fuel Selector – Check Tank

Oil Pressure – Check

Oil Temperature – Check

Fuel Pressure – Check

CHT – Check

EGT – Check

Fuel Flow – Check

Flight Controls – Free & Correct

Altimeter – Correct Altitude

Magnetometer – Correct Heading

Mixture – Rich

Trim – Aileron & Elevator Set for Takeoff

Brakes - Set

Throttle – 1800 RPM

Mag Check L&R

125 Max Drop, 50 Max Diff

Prop – Cycle

Prop – High RPM

Engine Instruments – Check

Throttle – IDLE

Throttle – 1000 RPM

Transponder - Set

Flaps – 0 Degrees

Comm#1: Tower – 133.100 TX & RX

Comm#2: Ground Crew – 123.45 RX

Fuel Pump On

Get Takeoff Clearance:

Man Tower, Experimental 997RV on first test flight, #1@ Runway XX

Takeoff:

Slow application of power

Check RPM

Right rudder

Abort if anything amiss

Rotate @ 55-60 KIAS

Small Wing Wag

Abort without positive control

Climb @ 90-110 KIAS

Climb to 1000 feet AGL before turn

Continue climb to 1800 MSL (shallow)

Level Off

Throttle 2300 RPM

Trim for level flight

Boost Pump Off

Cruise Check:

Oil Pressure – Verify

Oil Temperature – Verify

CHT – Verify

EGT – Verify

RPM – Verify

Manifold Pressure - Check

Fuel Pressure – Verify

Fuel Flow – Verify

Manifold Pressure - Verify

Flap Deployment Test, 1 notch only

Slow Flight Test

Landing:

Fuel Pump - ON

Airspeed – Below 95 KIAS

Prop – HIGH RPM

Flaps – 15 Degrees

Trim – As required

Approach Speed – 80 KIAS

Final Speed – 70 KIAS

After Landing:

Flaps - Up

Fuel Pump – OFF

Trim – RESET for Takeoff

Shutdown:

Flaps – DOWN FULL

Seat Heat – OFF

Switches – OFF

Fuel Truck – CALL?

Avionics – OFF

Alternator – OFF

Throttle – 1000 RPM

Mixture – IDLE CUTOFF

Magnetos – BOTH OFF

Master Switch – OFF

Fuel Selector - OFF