RV7A CONDITION INSPECTION REPORT

| OWNER James S. Clark IV | A/C TYPE RV7A | DATE 10/30/16 - |
| --- | --- | --- |
| SER. NO. J5C4 | N NUMBER 997RV | MANUFACT. DATE 10/2015 |
| TYPE INSPECTION Annual Condition Inspection | TACH TIME / HOBBS TIME 121.1 / 135.9 | TOTAL A/C TIME TACH/HOBBS 121.1 / 135.9 |

| PREP WORK |  |  | ##### INSP. |
| --- | --- | --- | --- |
|  |  | CLEAR TABLE FOR ALL REMOVED HARDWARE | ##### JSC4 |
|  |  | CONSUMABLES ORDERED | ##### JSC4 |
| A. PROP. GROUP |  |  |  |
|  | 1 | REMOVE SPINNER NOTING ORIENTATION, INSPECT SPINNER, INSPECT BULKHEADS FOR DEFECTS, CRACKS, LOOSE/SMOKING RIVETS | JSC4 |
|  | 2 | INSPECT BLADES FOR SECURITY, NICKS, CRACKS, OR OTHER DAMAGE | JSC4 |
|  | 3 | INSPECT PROP HUB FOR EVIDENCE OF DAMAGE, CRACKS, OR CORROSION | JSC4 |
|  | 4 | CHECK FOR GREASE AND OIL LEAKS | JSC4 |
|  | 5 | CHECK ALL MOUNT HARDWARE FOR DEFECTS AND SECURITY | JSC4 |
|  | 6 | CHECK ATTACH BOLTS FOR PROPER TORQUE (TBD) AND SAFETY WIRE | JSC4 |
|  | 7 | ###### CONSTANT SPEED | JSC4 |
|  |  | CHECK BLADES FOR TIGHTNESS IN HUB | JSC4 |
|  |  | REMOVE PROP, REMOVE SLUDGE | SKIP |
|  |  | LUBRICATE AS PER MANUAL | ###### JSC4 |
|  |  | INSPECT COMPLETE ASSEMBLY | JSC4 |
|  | 8 | REINSTALL SPINNER | JSC4 |
| B. ENGINE GROUP |  |  |  |
|  | 1 | REMOVE ENGINE COWLS | JSC4 |
|  | 2 | CLEAN COWLS, INSPECT FOR CRACKS, BROKEN HINGE EYELETS, MISSING FASTENERS | JSC4 |
|  |  | ###### REMOVE TOP SPARK PLUGS & IGNITION LEADS (3/4” Crows Foot & 7/8” PLUG SOCKET) | JSC4 |
|  | 3 | ###### COMPRESSION TEST /80 HOT_______COLD__Y__1._76__2._78__3.__78__4.__78__ | JSC4 |
|  | 4 | DRAIN OIL | JSC4 |
|  | 5 | REMOVE AND INSPECT OIL SCREEN FOR DEBRIS AND CLEAN | JSC4 |
|  | 6 | REMOVE AND REPLACE OIL FILTER AND SAFETY WIRE OIL FILTER, CUT OLD FILTER | JSC4 |
|  | 7 | CHECK OIL SENDER UNIT FOR LEAKS AND SECURITY | JSC4 |
|  | 8 | CLEAN AND CHECK OIL COOLER FOR CRACKS AND SECURITY | JSC4 |
|  | 9 | CHECK OIL COOLER MOUNTINGS & BAFFLES FOR CRACKS AND SECURITY | JSC4 |
|  | 10 | CHECK AND CLEAN FUEL FILTER | JSC4 |
|  | 11 | INSPECT FUEL INJECTOR LINES / NOZZELS FOR DEFECTS AND SECURITY | JSC4 |
|  | 12 | CHECK FUEL SYSTEM FOR EVIDENCE OF LEAKAGE, DETERIORATION, AND SECURITY | JSC4 |
|  | 13 | CHECK FUEL LINES FOR AGE, LEAKS, AND SECURITY | JSC4 |
|  | 14 | CHECK MECHANICAL FUEL PUMP INLET/OUTLET FITTINGS ARE TIGHT AND SECURE | JSC4 |
|  | 15 | CHECK FUEL INJECTION INLET SCREENS | JSC4 |
|  | 16 | CHECK ALL OIL LINES FOR AGE, DETERIORATION, LEAKS AND SECURITY | JSC4 |
|  | 17 | SERVICE / REPLACE AIR FILTER, INPECT AIRBOX FOR CRACKS, SECURITY | JSC4 |
|  | 18 | CHECK HEATER AND CABIN HEAT CONTROL | JSC4 |
|  | 19 | CHECK MAGS FOR PROPER INSTALLATION, SECURITY AND OIL LEAKAGE | JSC4 |
|  | 20 | CHECK EXISTING MAG TIMING LEFT__24BTDC____RIGHT__25BTDC____ | JSC4 |
|  | 21 | REMOVE / INSTALL MAG FOR 500 HR CHECK IF REQUIRED | JSC4 |
|  | 22 | ADJUST MAG TIMING LEFT___SKIP___RIGHT___SKIP___ | JSC4 |
|  | 23 | CHECK MAG POINTS | SKIP |
|  | 24 | CHECK DISTRIBUTOR BLOCKS FOR CRACKS | SKIP |
|  |  | BURNED AREA | SKIP |
|  |  | CORROSION | SKIP |
|  |  | HEIGHT OF CONTACT SPRINGS | SKIP |
|  | 25 | CHECK IGNITION HARNESS AND INSULATORS, P-LEADS FOR SECURITY | HPT |
|  | 26 | SERVICE OR REPLACE SPARK PLUGS, GAP 0.016 TO 0.021 1B: 0.018 (Original) 1T: 0.018 (Newer Tempest) 2B: 0.018 (Original) 2T: 0.018 (Newer Tempest) 3B: 0.019 (Original) 3T: 0.018 (Newer Tempest) 4B: 0.018 (Original) 4T: 0.018 (Newer Tempest) Install bottom plugs to 360-420 inch lbs torque (30-35 foot lbs) | JSC4 |
|  | 27 | CHECK CONDITION OF ALTERNATOR, BELT AND WIRING, CHECK BELT TENSION | DWH |
|  | 28 | CHECK CONDITION OF STARTER AND CONNECTIONS | JSC4 |
|  | 29 | INSPECT EXHAUST SYSTEM FOR LEAKS, MOUNTS FOR SECURITY, CRACKS, TORQUE (TBD) | DWH |
|  |  | APPLY MOUSE MILK TO EXHAUST PIPE BALL JOINTS | DWH |
|  | 30 | INSPECT MUFFLERS, HEAT SHROUD, SCAT TUBING | ###### DWH |
|  | 31 | CHECK ENGINE BAFFLES AND BAFFLE ATTACHMENTS | DWH |
|  | 32 | CHECK BREATHER TUBE FOR OBSTRUCTIONS, SECURITY | DWH |
|  | 33 | CHECK CRANKCSE FOR LEAKS, CRACKS, LOOSE FASTENERS, ETC. | JSC4 |
|  | 34 | CHECK ENGINE MOUNTS FOR SECURITY, CHECK FIREWALL ATTACHMENT NUTS/BOLTS | JSC4 |
|  | 35 | CHECK ENGINE MOUNT BUSHINGS FOR DETERIORATION | DWH |
|  | 36 | CHECK FIREWALL SEALS | DWH |
|  | 37 | CHECK THROTTLE, MIXTURE, ALTERNATE AIR, PROP CONTROLS FOR SECURITY AND PROPER OPERATION. INSPECT ALL ATTACH HARDWARE FOR SECURITY | JSC4 |
|  | 38 | CHECK ALL INLET CLAMPS, GASKETS AND INLET TUBE BOLT TORQUE | JSC4 |
|  | 39 | INSPECT GENERAL CONDITION OF ENGINE | JSC4 |
|  | 40 | FILL ENGINE OIL..TYPE W15W-50 QUARTS 6 | DWH |
|  | 41 | CLEAN ENGINE | DWH |
|  | 42 | LUBRICATE CONTROLS | JSC4 |
|  | 43 | CHECK BATTERY TERMINALS ARE TIGHT, CLEAN. CABLES FOR DAMAGE, SECURITY | JSC4 |
|  | 44 | CHECK ENGINE GROUND CABLES FOR ATTACHMENT AND SECURITY | JSC4 |
| C. CABIN GROUP |  |  |  |
|  | 1 | INSPECT CANOPY HINGES, STRUTS, LATCHES, LOCKING MECHANISM, OPERATION | JSC4 |
|  | 2 | INSPECT CANOPY FRAME AND GLASS FOR GENERAL CONDITON | JSC4 |
|  | 3 | CHECK SEAT, SEAT BELTS, ATTACHMENTS AND BOLTS | DWH |
|  | 4 | CHECK ELEVATOR & AILERON TRIM OPERATION | JSC4 |
|  | 5 | CHECK FUEL ON-OFF VALVE FOR PROPER OPERATION AND MARKING | JSC4 |
|  | 6 | CHECK FUEL SYSTEM LINES, PUMP, B-NUTS FOR SECURITY, CHAFING, LEAKS. REMOVE FUEL FILTER AND CLEAN. CHECK FOR LEAKS (PUMP ON) | JSC4 |
|  | 7 | CHECK OPERATION OF FUEL DRAINS | JSC4 |
|  | 8 | CHECK OPERATION OF INTERIOR LIGHTING | DWH |
|  | 9 | CHECK RUDDER PEDALS, CABLES, CHECK TUBES AND PEDALS FOR CRACKS, SECURITY | DWH |
|  | 10 | CHECK CONTROL STICK,BEARINGS, PUSHRODS FOR OPERATION AND SECURITY | DWH |
|  | 11 | CHECK INSTRUMENTS, LINES, WIRES FOR PROPER INSTALLATION AND SECURITY | HPT |
|  | 12 | CHECK ALTIMETER / TRANSPONDER CERTIFICATION DATE DONE 10/1/2015, DUE 10/2017 | JSC4 |
|  | 13 | CHECK MASTER CYLINDERS FOR PROPER INSTALLATION, LEAKS, SECURITY | JSC4 |
|  | 14 | CHECK CONTROL STICKS, BOLTS, JAM NUTS, TORQUE TUBES FOR PROPER INSTALLTION AND SECURITY. LUBRICATE ALL BEARINGS AND BUSHINGS | JSC4 |
|  |  | REMOVE SEATS | DWH |
|  |  | REMOVE SEATING AREA CARPET | DWH |
|  |  | INSPECT PITCH BELLCRANK | DWH |
|  |  | REMOVE SEAT PANS | JSC4 |
|  |  | VACUUM UNDER SEATS | JSC4 |
|  |  | VACUUM FOOTWELL AREA | JSC4 |
| ###### D. FUSELAGE GROUP |  |  |  |
|  | 1 | REMOVE ALL INSPECTION PANELS INCLUDING TAIL FAIRINGS | DWH |
|  | 2 | CHECK GENERAL CONDITION OF SKIN | JSC4 |
|  | 3 | REMOVE BAGGAGE COMPARTMENT CARPET, SIDE COVERS AND BAGGAGE BULKHEAD | DWH |
|  | 4 | CHECK WIRING, HOSES, ETC. FOR DAMAGE AND SECURITY | DWH |
|  | 5 | CHECK ANTENNA WIRES AND CONNECTIONS | DWH |
|  | 6 | CHECK AND SERVICE E.L.T. BATTERY INSTALLED 10/2015 BATTERY INSPECTED 11/23/2016 BATTERY GOOD UNTIL___2025________________ | JSC4 |
|  | 7 | CHECK BULKHEAD AND STRINGERS FOR DAMAGE | DWH |
|  | 9 | CHECK EMPENNAGE SURFACES FOR DAMAGE | DWH |
|  | 10 | CHECK RUDDER HINGES, HORN, AND ATTACHMENTS | JSC4 |
|  | 11 | CHECK VERTICAL FIN ATTACHMENTS | JSC4 |
|  | 12 | CHECK ELEVATOR HINGES, HORN AND ATTACHMENTS | JSC4 |
|  | 13 | CHECK THAT ELEVATOR TRAVEL IS WITHIN LIMITS: ACTUAL UP_______DOWN________ UP MAX: 30, UP MIN: 25, DOWN MAX: 25, DOWN MIN: 20 DEGREES | JSC4 |
|  | 14 | CHECK HORIZONTAL STABILIZER ATTACHMENTS | JSC4 |
|  | 15 | CHECK ELEVATOR TRIM FOR INSTALLATION, SECURITY AND OPERATION | JSC4 |
|  | 16 | CHECK AUTOPILOT SERVO INSTALLATION | JSC4 |
|  | 17 | CHECK OPERATION OF REAR STROBE AND POSITION LIGHT | JSC4 |
|  | 18 | RE-INSTALL PANELS AND TAIL FAIRING | JSC4 |
| ###### E. WING GROUP |  |  |  |
|  | 1 | CHECK WINGTIP, AILERON, FLAP ALIGNMENT | JSC4 |
|  | 2 | REMOVE INSPECTION PLATES, ROOT FAIRINGS, AND WINGTIPS | JSC4 |
|  | 3 | CHECK WING SKINS FOR DAMAGE, DISTORTION, SMOKING/LOOSE RIVETS | JSC4 |
|  | 4 | CHECK WING TIPS FOR DAMAGE | JSC4 |
|  | 5 | CHECK AILERON HINGES, PUSHRODS FOR OPERATION | JSC4 |
|  | 6 | CHECK ROLL AUTOPILOT INSTALLATION FOR SECURITY & PROPER INSTALLATION | JSC4 |
|  | 7 | CHECK AILERON TRAVEL IS WITHIN LIMITS: MAX UP: 32, MIN UP: 25, MAX DOWN: 17, MIN DOWN: 15 DEGREES ACTUAL RIGHT UP:___32___ ACTUAL RIGHT DOWN:__17____ ACTUAL LEFT UP:____30___ ACTUAL LEFT DOWN:___17_____ | JSC4 |
|  | 8 | CHECK FLAP CONDITION, SECURITY, LUBRICATE HINGES AND BEARINGS | JSC4 |
|  | 9 | CHECK FLAP OPERATION, INSPECT AND CHECK FLAP ACTUATOR, FLAP ACTUATOR ROD BEARINGS, FLAP INDICATOR SECURITY, AND FLAP MECHANISM SAFETY WIRE | JSC4 |
|  | 10 | CHECK WING SPAR ATTACH BOLTS (MAIN AND AFT SPAR) FOR PROPER TORQUE (TBD) | JSC4 |
|  | 11 | CHECK HEATED PITOT AND LINES FOR SECURITY, CHAFING AND OPERATION | JSC4 |
|  | 12 | CHECK WINGTIPS FOR DAMAGE AND SECURITY | JSC4 |
|  | 13 | CHECK OPERATION OF LANDING LIGHTS | JSC4 |
|  | 14 | CHECK OPERATION OF STROBE LIGHTS | JSC4 |
|  | 15 | CHECK OPERATION OF POSITION/NAV LIGHTS | JSC4 |
|  | 16 | RE-INSTALL INSPECTION PANELS AND FAIRINGS | JSC4 |
|  | 17 | CHECK CONDITION OF FUEL CAPS, O RINGS, AND FUEL SUMP DRAINS | JSC4 |
| F. LANDING GEAR |  |  |  |
|  | 1 | PUT AIRCRAFT ON JACKS | JSC4 |
|  | 2 | REMOVE WHEEL PANTS AND GEAR FAIRINGS | JSC4 |
|  | 3 | CHECK TIRES FOR WEAR AND DAMAGE | JSC4 |
|  | 4 | REMOVE WHEELS INSPECT FOR CRACKS, CORROSION, CONDITION OF BOLTS | JSC4 |
|  | 5 | CHECK TIRE PRESSURES LEFT___40___NOSE___40___RIGHT___40___ | JSC4 |
|  | 6 | CHECK BRAKE LININGS AND DISCS FOR WEAR | JSC4 |
|  | 7 | CHECK BRAKE LINES, CALIPERS FOR LEAKS, SECURITY AND CONDITION | JSC4 |
|  | 8 | CHECK WHEEL FAIRINGS AND MOUNTING PLATES FOR CRACKS AND SECURITY, CHECK ALL BOLTS FOR PROPER SECURITY/TORQUE | JSC4 |
|  | 8 | CHECK LANDING GEAR STRUTS FOR CRACKS AND LOOSENESS/SECURITY | JSC4 |
|  | 9 | CLEAN AND LUBRICATE MAIN WHEEL BEARINGS, AND RE-INSTALL MAIN WHEELS | JSC4 |
|  |  | CHECK ALL LANDING GEAR UPPER RETAINING BOLT TORQUE (TBD) | JSC4 |
|  | 10 | RE-INSTALL WHEELS & BRAKES | JSC4 |
|  | 11 | CHECK NOSE WHEEL BEARING ADJUSTMENT | JSC4 |
|  | 12 | CHECK BREAKOUT FORCE ON NOSE WHEEL: DESIRED LBS: 22 AT AXEL ACTUAL LBS:__22+____ | JSC4 |
|  | 13 | CHECK BRAKE CYLINDERS FOR OPERATION AND LEAKS | JSC4 |
|  | 14 | CHECK BRAKE FLUID LEVEL. 7/16” TYPE OF FLUID BEING USED__ROYCO 782______ | JSC4 |
|  | 15 | INSTALL WHEEL PANTS AND GEAR FAIRINGS | JSC4 |
|  | 16 | REMOVE AIRCRAFT FROM JACKS | JSC4 |
| G. OPERATION CHECK |  |  |  |
|  | 1 | CHECK BOOST PUMP PRESSURE | JSC4 |
|  | 2 | CHECK ALTERNATOR OUTPUT | JSC4 |
|  | 3 | CHECK AVIONICS EQUIPMENT OPERATION | JSC4 |
|  | 4 | CHECK OIL PRESSURE | JSC4 |
|  | 5 | CHECK MANIFOLD PRESSURE | JSC4 |
|  | 6 | CHECK PROPELLER OPERATION AND SMOOTHNESS | JSC4 |
|  | 7 | CHECK MAG DROP AT 1800 RPM LEFT___90___RIGHT___90___ | JSC4 |
|  | 8 | CHECK STATIC R.P.M | JSC4 |
|  | 9 | CHECK IDLE R.P.M. | JSC4 |
|  | 10 | CHECK MAG SWITCH WIRES FOR GROUNDING | JSC4 |
|  | 11 | CHECK THROTTLE AND MIXTURE CONTROL OPERATION | JSC4 |
|  | 12 | CHECK IDLE MIXTURE | JSC4 |
|  | 13 | LEAK CHECK ENGINE | JSC4 |
|  | 14 | RE-INSTALL COWLS | JSC4 |
|  | 15 | CHECK FLIGHT CONTROL OPERATION (AILERON, ELEVATOR, RUDDER, TRIM) | JSC4 |
| H. GENERAL |  |  |  |
|  | 1. | AIRCRAFT CONFORMS TO FAA SPECIFICATION | N/A |
|  | 2. | AIRCRAFT PAPERS IN PROPER ORDER A. REGISTRATION | JSC4 |
|  |  | B. AIRWORTHINESS | JSC4 |
|  |  | C. OPERATING LIMITS | JSC4 |
|  |  | AD INSPECTIONS AIRFRAME & ENGINE & PROP | JSC4 |
|  |  | VERIFY SERVICE BULLETINS COMPLIANCE | JSC4 |
|  |  | LOGBOOK ENTRIES, AIRFRAME, ENGINE, PROP |  |
| Notes: |  | Found wet B-Nut on rear side of fuel filter, retightened | JSC4 |
|  |  | Found loose spare AN365 nut under passenger seat pan, removed | JSC4 |
|  |  | Found rivet head scrap junk in footwell area, removed | JSC4 |
|  |  | Found plastic hose cap in footwell area, removed | JSC4 |
|  |  | Removed passenger stick, cut zip ties, separated wire harness | JSC4 |
|  |  | Removed left and right gear tower carpet covers | JSC4 |
|  |  | Found cracked engine baffle near oil cooler mount on #4 cylinder, added reinforcement | JSC4 |
|  |  | Each aileron trailing edge droops 3/16 when opposite aileron/flap/tip are in alignment, no change | JSC4 |
|  |  | Routed GDL cable to dash top through hole & installed grommet | JSC4 |
|  |  | Added pin #15 on roll servo via spare white wire (CWS Disc) and spliced it into green CWS Disc wires for sticks under right passenger seat pan | JSC4 |
|  |  | Added prop leading edge tape | JSC4 |
|  |  | Reinstalled passenger stick, zip-tied harness | JSC4 |
|  |  | Checked autopilot disconnect swtiches immediately affect both servos (on both sticks) | JSC4 |
|  |  | Performed AD 2015-19-07: Inspected fuel injector lines – No problems | JSC4 |
|  |  | Inspected Vans SB 14-02-05: Elevator Spar Cracks – Done – None found | JSC4 |
|  |  | Inspected Vans SB 14-01-31: HS Front spar bend cracks – Inspected – None found | JSC4 |
|  |  | Inspected Vans SB 16-03-28: aileron inboard wing spar attach cracks – Inspected – None found | JSC4 |

SIGNATURE OF INSPECTOR___________________________________________________________________

CERTIFICATE NUMBER____________Repairman # Pending: James S. Clark IV_____________________________________

DATE OF SIGN-OFF_______________12/10/16____________________________________________________________